Calibration

- Start by double-clicking on MaxTRAQ3D Cal Settings.reg and then select Yes when asked if you want to add the information to the registry.

- Start MaxTRAQ 3D

- Select Calibration

- Load the 2 reference files Ref0 and Ref1 by clicking on the Open button in the 2 camera windows



You can choose to open the digitized file (.maq) or the video (.avi) file.  If you open the AVI files, you will need to digitize/track the files by clicking on the Track Button

- Click on the “Use as Reference” button

- Load the 2 wand movement files, Wand0 and Wand1.  Again you can select to open the digitized files or the video (AVI) files.

- Click on the Calibrate tool button

- Click OK

3D Transformation

- Select Create 3D

- Load the 2 walking files, Walk0 and Walk1.

- Click on the 3D Transform tool button

- The 3D graphs should now display the 3D data

- The data can now be saved as either a C3D file or an ASCII file.  The ASCII file can be opened in MaxMATE for further analysis